實驗二

Time-of-day clock

學生: 謝旻錡 學號: 313512078 日期: 2025/03/12

- 實驗目的

熟悉latches、flip-flop及regist，並以此基礎組合出counter，並藉由counter的功能，實作一個循環24小時制的時鐘。

時鐘輸入輸出規格:

| Input |  | Output |  |
| --- | --- | --- | --- |
| SW9 | Select hour and minute | HEX5~4 | Hour |
| SW8 | Set time (positive edge) | HEX3~2 | Minute |
| SW7~0 | The value of setting time | HEX1~0 | Second |

- 實驗程式碼

STEP.1 我使用QUARTER的LPM (程式碼至於附件)，定義了second/ minute/ hour的counters，並配合frequency divider計算second/ minute/ hour時間。

STEP.2 計算出的值為二進制，所以需要使用BCD來轉換成10進制，這邊的BCD我使用Double Dabble演算法來轉換。

STEP.3 最後接上七段顯示器顯示時間。

- LMP_counters

圖 1. counters

| 變數 | 內容 |
| --- | --- |
| clk | 時鐘訊號 (50MHz) |
| reset, reset1, reset2 | counter歸零訊號 (1: rest; 0: nothing) |
| sec_Q, min_Q, Hr_Q | 記數數量 |

表格 1. Counter變數

我定義了三個counter，分別用於second, minute, hour 的clk訊號的記數。每20ns記數一次(clk: 50MHz)。

- 時間設定功能

圖 2. 設定時間

| 變數 | 內容 |
| --- | --- |
| select | 選擇設定hour或minute (1: hour; 0: minute) |
| set | 傳入設定時間 (positive edge) |
| set_time | 欲設定的時間 |
| SW8_N, SW8_P | SW8開關的現在值與上一個clk的開關值 |

表格 2. Time setting變數

宣告一個8bit的set_time 暫存器，儲存SW[7:0]開關欲設定的時間。

為了使 set是以正緣觸發的方式設定時間，我寫段邏輯電路 set = (~SW8_P) & SW8N; 。因此當SW8為0轉至1的瞬間，set的值為1，以此滿足正緣觸發的條件。

- 計時器 second/ minute/ hour timer

圖 3. Second timer

圖 4. Minute timer

圖 5. Hour timer

| 變數 | 內容 |
| --- | --- |
| clk | 時鐘訊號 (50MHz) |
| reset, reset1, reset2 | counter歸零訊號 (1: rest; 0: nothing) |
| sec_Q, min_Q, hr_Q | 記數數量 |
| sec, min, hr | 計時數值 |
| set_time | 欲設定的時間 |
| set | 傳入設定時間 |
| select | 選擇設定hour或minute (1: hour; 0: minute) |

表格 3. Timer變數

分別設定三個計時器來對second/ minute/ hour計時。

Second: 每當counter數了5×107次，為一秒。如果當sec數了59秒時，sec下一秒歸零。

Minute: 每當counter數了3×109次，為一分鐘。當min數了59秒，sec數了59秒時，下一秒min歸零。

Hour: 每當counter 數了2×1011次，為一小時。當hr數了23小時，min數了59秒，sec數了59秒時，下一秒hr歸零。

- BCD

圖 6. BCD of second

圖 7. BCD of minute

圖 8. BCD of hour

| 變數 | 內容 |
| --- | --- |
| bin, bin1, bin2 | 儲存sec/ min/ hr的暫存器 |
| shift_reg | 用於sec的BCD計算的暫存空間 |
| shift_reg1 | 用於min的BCD計算的暫存空間 |
| shift_reg2 | 用於hr的BCD計算的暫存空間 |
| sec_bcd_t, sec_bcd_o | 秒 轉換後的10進制值 |
| min_bcd_t, min_bcd_o | 分 轉換後的10進制值 |
| hr_bcd_t, hr_bcd_o | 時 轉換後的10進制值 |

sec/ min/ hr裡儲存的時間為二進制形式，因此我藉由使用double dabble的演算法，將其轉換為二進制。

Double dabble演算法:

遞推關係

- 初始條件（當 i=0時）：

D(0)=(0,0,...,0,B)

BCD 欄位初始化為 0。

- 主要轉換步驟（第 i 次迭代）：

若 Dj​(i)≥5, 則 Dj​(i)=Dj​(i)+3,∀j∈[0,k-1]

然後整體左移一位：

D(i+1)=(Dk-1​(i)≪1,Dk-2​(i)≪1,...,D0​(i)≪1,R(i)≪1+bn-1-i​)

這個過程會持續 n 次，最終 D(n)即為 BCD 的結果。

- 7段顯示器

圖 9. 7段顯示器-秒

圖 10. 段顯示器-分

圖 11. 段顯示器-時

| 變數 | 內容 |
| --- | --- |
| HEX0, HEX1 | 秒的10進制顯示 |
| HEX2, HEX3 | 分的10進制顯示 |
| HEX4, HEX5 | 時的10進制顯示 |

- 實驗結果照片(optional)

- RTL布局(optional)

- 問題與討論

  - RTL線路過大，我認為是因為我設計時，使用了三個counters。可以改成只用一個counter來實作，減少電路量。

  - 如果改用一個counter製作，我會把timer在單獨拉出一個獨立的模組。這樣可以在修改時，簡潔易點。

- 附件

/*part7 day clock*/

module part7 (SW, HEX0, HEX1, HEX2, HEX3, HEX4, HEX5, CLOCK_50 ); input CLOCK_50; input [9:0] SW; output [6:0] HEX0, HEX1, HEX2, HEX3, HEX4, HEX5; reg reset, reset1, reset2; wire clock; wire [25:0] sec_Q; wire [31:0] min_Q; wire [37:0] hr_Q; reg [5:0] sec; reg [7:0] min; reg [7:0] hr; reg [7:0] set_time; //二進制 assign clk = CLOCK_50; //define sec/min/hr counters part7_counter sec_cr(clk, reset, sec_Q); part7_counter1 min_cr(clk, reset1, min_Q); part7_counter2 hr_cr(clk, reset2, hr_Q); //SW reg SW8_N, SW8_P, set, select; always @(*)begin set = (~SW8_P) & SW8_N; select = SW[9]; end always @(posedge clk)begin SW8_N <= SW[8]; SW8_P <= SW8_N; set_time[7:0] <= SW[7:0]; end //========== sec/min/hr time ======== //sec_ 0~59 always @(posedge clk)begin if(sec_Q == 26'd49999998) reset <= 1; else if(sec_Q >= 26'd49999999)begin if(sec >= 10'd59)begin sec <= 0; reset <= 0; end else begin sec <= sec + 1; reset <= 0; end end else reset <= 0; end //min_ 0~59 always @(posedge clk)begin if(set && ~select && (set_time < 60)) min <= set_time; else begin if(min_Q == 32'd2999999998) reset1 <= 1; else if(min_Q >= 32'd2999999999)begin if(min >= 10'd59)begin min <= 0; reset1 <= 0; end else begin min <= min + 1; reset1 <= 0; end end else reset1 <= 0; end end //hr_ 0~24 always @(posedge clk)begin if(hr == 23 && min == 59 && sec == 59 && sec_Q == 26'd49999999) hr <= 0; else if( min == 59 && sec == 59 && sec_Q == 26'd49999999) hr <= hr +1; else if(set && select && (set_time < 24)) hr <= set_time; else begin if(hr_Q == 38'd179999999998) reset2 <= 1; else if(hr_Q >= 38'd179999999999)begin if(hr >= 10'd24)begin hr <= 0; reset2 <= 0; end else begin hr <= hr + 1; reset2 <= 0; end end else reset2 <= 0; end end //==================== BCD ========================== //BCD sec wire [5:0] bin; assign bin = sec; integer i; reg [13:0] shift_reg; reg [3:0] sec_bcd_t, sec_bcd_o; always @(*) begin shift_reg = {8'b0, bin}; // 初始化轉換暫存器 for (i = 0; i < 6; i = i + 1) begin if (shift_reg[13:10] >= 5) shift_reg[13:10] = shift_reg[13:10] + 3; if (shift_reg[9:6] >= 5) shift_reg[9:6] = shift_reg[9:6] + 3; shift_reg = shift_reg << 1; // 左移 1 位 end sec_bcd_t = shift_reg[13:10]; // 十位 sec_bcd_o = shift_reg[9:6]; // 個位 end //BCD min wire [7:0] bin1; assign bin1 = min; integer j; reg [13:0] shift_reg1; reg [3:0] min_bcd_t, min_bcd_o; always @(*) begin shift_reg1 = {6'b0, bin1}; // 初始化轉換暫存器 for (j = 0; j < 6; j = j + 1) begin if (shift_reg1[13:10] >= 5) shift_reg1[13:10] = shift_reg1[13:10] + 3; if (shift_reg1[9:6] >= 5) shift_reg1[9:6] = shift_reg1[9:6] + 3; shift_reg1 = shift_reg1 << 1; // 左移 1 位 end min_bcd_t = shift_reg1[13:10]; // 十位 min_bcd_o = shift_reg1[9:6]; // 個位 end //BCD hr wire [7:0] bin2; assign bin2 = hr; integer k; reg [12:0] shift_reg2; reg [3:0] hr_bcd_t, hr_bcd_o; always @(*) begin shift_reg2 = {5'b0, bin2}; // 初始化轉換暫存器 for (k = 0; k < 5; k = k + 1) begin if (shift_reg2[12:9] >= 5) shift_reg2[12:9] = shift_reg2[12:9] + 3; if (shift_reg2[8:5] >= 5) shift_reg2[8:5] = shift_reg2[8:5] + 3; shift_reg2 = shift_reg2 << 1; // 左移 1 位 end hr_bcd_t = shift_reg2[12:9]; // 十位 hr_bcd_o = shift_reg2[8:5]; // 個位 end //================ HEX5~0 ========================= //HEX0, HEX1 reg [6:0] reg_HEX0, reg_HEX1; assign HEX0 = reg_HEX0; assign HEX1 = reg_HEX1; always @(*)begin case (sec_bcd_o) 4'b0000: reg_HEX0 = 7'b1000000; // 0 4'b0001: reg_HEX0 = 7'b1111001; // 1 4'b0010: reg_HEX0 = 7'b0100100; // 2 4'b0011: reg_HEX0 = 7'b0110000; // 3 4'b0100: reg_HEX0 = 7'b0011001; // 4 4'b0101: reg_HEX0 = 7'b0010010; // 5 4'b0110: reg_HEX0 = 7'b0000010; // 6 4'b0111: reg_HEX0 = 7'b1111000; // 7 4'b1000: reg_HEX0 = 7'b0000000; // 8 4'b1001: reg_HEX0 = 7'b0010000; // 9 default: reg_HEX0 = 7'b1000000; // 0 endcase end always @(*)begin case (sec_bcd_t) 4'b0000: reg_HEX1 = 7'b1000000; // 0 4'b0001: reg_HEX1 = 7'b1111001; // 1 4'b0010: reg_HEX1 = 7'b0100100; // 2 4'b0011: reg_HEX1 = 7'b0110000; // 3 4'b0100: reg_HEX1 = 7'b0011001; // 4 4'b0101: reg_HEX1 = 7'b0010010; // 5 4'b0110: reg_HEX1 = 7'b0000010; // 6 4'b0111: reg_HEX1 = 7'b1111000; // 7 4'b1000: reg_HEX1 = 7'b0000000; // 8 4'b1001: reg_HEX1 = 7'b0010000; // 9 default: reg_HEX1 = 7'b1000000; // 0 endcase end //HEX2, HEX3 reg [6:0] reg_HEX2, reg_HEX3; assign HEX2 = reg_HEX2; assign HEX3 = reg_HEX3; always @(*)begin case (min_bcd_o) 4'b0000: reg_HEX2 = 7'b1000000; // 0 4'b0001: reg_HEX2 = 7'b1111001; // 1 4'b0010: reg_HEX2 = 7'b0100100; // 2 4'b0011: reg_HEX2 = 7'b0110000; // 3 4'b0100: reg_HEX2 = 7'b0011001; // 4 4'b0101: reg_HEX2 = 7'b0010010; // 5 4'b0110: reg_HEX2 = 7'b0000010; // 6 4'b0111: reg_HEX2 = 7'b1111000; // 7 4'b1000: reg_HEX2 = 7'b0000000; // 8 4'b1001: reg_HEX2 = 7'b0010000; // 9 default: reg_HEX2 = 7'b1000000; // 0 endcase end always @(*)begin case (min_bcd_t) 4'b0000: reg_HEX3 = 7'b1000000; // 0 4'b0001: reg_HEX3 = 7'b1111001; // 1 4'b0010: reg_HEX3 = 7'b0100100; // 2 4'b0011: reg_HEX3 = 7'b0110000; // 3 4'b0100: reg_HEX3 = 7'b0011001; // 4 4'b0101: reg_HEX3 = 7'b0010010; // 5 4'b0110: reg_HEX3 = 7'b0000010; // 6 4'b0111: reg_HEX3 = 7'b1111000; // 7 4'b1000: reg_HEX3 = 7'b0000000; // 8 4'b1001: reg_HEX3 = 7'b0010000; // 9 default: reg_HEX3 = 7'b1000000; // 0 endcase end //HEX4, HEX5 reg [6:0] reg_HEX4, reg_HEX5; assign HEX4 = reg_HEX4; assign HEX5 = reg_HEX5; always @(*)begin case (hr_bcd_o) 4'b0000: reg_HEX4 = 7'b1000000; // 0 4'b0001: reg_HEX4 = 7'b1111001; // 1 4'b0010: reg_HEX4 = 7'b0100100; // 2 4'b0011: reg_HEX4 = 7'b0110000; // 3 4'b0100: reg_HEX4 = 7'b0011001; // 4 4'b0101: reg_HEX4 = 7'b0010010; // 5 4'b0110: reg_HEX4 = 7'b0000010; // 6 4'b0111: reg_HEX4 = 7'b1111000; // 7 4'b1000: reg_HEX4 = 7'b0000000; // 8 4'b1001: reg_HEX4 = 7'b0010000; // 9 default: reg_HEX4 = 7'b1000000; // 0 endcase end always @(*)begin case (hr_bcd_t) 4'b0000: reg_HEX5 = 7'b1000000; // 0 4'b0001: reg_HEX5 = 7'b1111001; // 1 4'b0010: reg_HEX5 = 7'b0100100; // 2 4'b0011: reg_HEX5 = 7'b0110000; // 3 4'b0100: reg_HEX5 = 7'b0011001; // 4 4'b0101: reg_HEX5 = 7'b0010010; // 5 4'b0110: reg_HEX5 = 7'b0000010; // 6 4'b0111: reg_HEX5 = 7'b1111000; // 7 4'b1000: reg_HEX5 = 7'b0000000; // 8 4'b1001: reg_HEX5 = 7'b0010000; // 9 default: reg_HEX5 = 7'b1000000; // 0 endcase end endmodule

part7_counter.v

// megafunction wizard: %LPM_COUNTER% // GENERATION: STANDARD // VERSION: WM1.0 // MODULE: LPM_COUNTER // ============================================================ // File Name: part7_counter.v // Megafunction Name(s): // LPM_COUNTER // // Simulation Library Files(s): // lpm // ============================================================ // ************************************************************ // THIS IS A WIZARD-GENERATED FILE. DO NOT EDIT THIS FILE! // // 13.1.0 Build 162 10/23/2013 SJ Full Version // ************************************************************ //Copyright (C) 1991-2013 Altera Corporation //Your use of Altera Corporation's design tools, logic functions //and other software and tools, and its AMPP partner logic //functions, and any output files from any of the foregoing //(including device programming or simulation files), and any //associated documentation or information are expressly subject //to the terms and conditions of the Altera Program License //Subscription Agreement, Altera MegaCore Function License //Agreement, or other applicable license agreement, including, //without limitation, that your use is for the sole purpose of //programming logic devices manufactured by Altera and sold by //Altera or its authorized distributors. Please refer to the //applicable agreement for further details. // synopsys translate_off `timescale 1 ps / 1 ps // synopsys translate_on module part7_counter ( clock, sclr, q); input clock; input sclr; output [25:0] q; wire [25:0] sub_wire0; wire [25:0] q = sub_wire0[25:0]; lpm_counter LPM_COUNTER_component ( .clock (clock), .sclr (sclr), .q (sub_wire0), .aclr (1'b0), .aload (1'b0), .aset (1'b0), .cin (1'b1), .clk_en (1'b1), .cnt_en (1'b1), .cout (), .data ({26{1'b0}}), .eq (), .sload (1'b0), .sset (1'b0), .updown (1'b1)); defparam LPM_COUNTER_component.lpm_direction = "UP", LPM_COUNTER_component.lpm_port_updown = "PORT_UNUSED", LPM_COUNTER_component.lpm_type = "LPM_COUNTER", LPM_COUNTER_component.lpm_width = 26; endmodule // ============================================================ // CNX file retrieval info // ============================================================ // Retrieval info: PRIVATE: ACLR NUMERIC "0" // Retrieval info: PRIVATE: ALOAD NUMERIC "0" // Retrieval info: PRIVATE: ASET NUMERIC "0" // Retrieval info: PRIVATE: ASET_ALL1 NUMERIC "1" // Retrieval info: PRIVATE: CLK_EN NUMERIC "0" // Retrieval info: PRIVATE: CNT_EN NUMERIC "0" // Retrieval info: PRIVATE: CarryIn NUMERIC "0" // Retrieval info: PRIVATE: CarryOut NUMERIC "0" // Retrieval info: PRIVATE: Direction NUMERIC "0" // Retrieval info: PRIVATE: INTENDED_DEVICE_FAMILY STRING "Cyclone V" // Retrieval info: PRIVATE: ModulusCounter NUMERIC "0" // Retrieval info: PRIVATE: ModulusValue NUMERIC "0" // Retrieval info: PRIVATE: SCLR NUMERIC "1" // Retrieval info: PRIVATE: SLOAD NUMERIC "0" // Retrieval info: PRIVATE: SSET NUMERIC "0" // Retrieval info: PRIVATE: SSET_ALL1 NUMERIC "1" // Retrieval info: PRIVATE: SYNTH_WRAPPER_GEN_POSTFIX STRING "0" // Retrieval info: PRIVATE: nBit NUMERIC "26" // Retrieval info: PRIVATE: new_diagram STRING "1" // Retrieval info: LIBRARY: lpm lpm.lpm_components.all // Retrieval info: CONSTANT: LPM_DIRECTION STRING "UP" // Retrieval info: CONSTANT: LPM_PORT_UPDOWN STRING "PORT_UNUSED" // Retrieval info: CONSTANT: LPM_TYPE STRING "LPM_COUNTER" // Retrieval info: CONSTANT: LPM_WIDTH NUMERIC "26" // Retrieval info: USED_PORT: clock 0 0 0 0 INPUT NODEFVAL "clock" // Retrieval info: USED_PORT: q 0 0 26 0 OUTPUT NODEFVAL "q[25..0]" // Retrieval info: USED_PORT: sclr 0 0 0 0 INPUT NODEFVAL "sclr" // Retrieval info: CONNECT: @clock 0 0 0 0 clock 0 0 0 0 // Retrieval info: CONNECT: @sclr 0 0 0 0 sclr 0 0 0 0 // Retrieval info: CONNECT: q 0 0 26 0 @q 0 0 26 0 // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter.v TRUE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter.inc FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter.cmp FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter.bsf FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter_inst.v FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter_bb.v TRUE // Retrieval info: LIB_FILE: lpm

part7_counter1.v

/ megafunction wizard: %LPM_COUNTER% // GENERATION: STANDARD // VERSION: WM1.0 // MODULE: LPM_COUNTER // ============================================================ // File Name: part7_counter1.v // Megafunction Name(s): // LPM_COUNTER // // Simulation Library Files(s): // lpm // ============================================================ // ************************************************************ // THIS IS A WIZARD-GENERATED FILE. DO NOT EDIT THIS FILE! // // 13.1.0 Build 162 10/23/2013 SJ Full Version // ************************************************************ //Copyright (C) 1991-2013 Altera Corporation //Your use of Altera Corporation's design tools, logic functions //and other software and tools, and its AMPP partner logic //functions, and any output files from any of the foregoing //(including device programming or simulation files), and any //associated documentation or information are expressly subject //to the terms and conditions of the Altera Program License //Subscription Agreement, Altera MegaCore Function License //Agreement, or other applicable license agreement, including, //without limitation, that your use is for the sole purpose of //programming logic devices manufactured by Altera and sold by //Altera or its authorized distributors. Please refer to the //applicable agreement for further details. // synopsys translate_off `timescale 1 ps / 1 ps // synopsys translate_on module part7_counter1 ( clock, sclr, q); input clock; input sclr; output [31:0] q; wire [31:0] sub_wire0; wire [31:0] q = sub_wire0[31:0]; lpm_counter LPM_COUNTER_component ( .clock (clock), .sclr (sclr), .q (sub_wire0), .aclr (1'b0), .aload (1'b0), .aset (1'b0), .cin (1'b1), .clk_en (1'b1), .cnt_en (1'b1), .cout (), .data ({32{1'b0}}), .eq (), .sload (1'b0), .sset (1'b0), .updown (1'b1)); defparam LPM_COUNTER_component.lpm_direction = "UP", LPM_COUNTER_component.lpm_port_updown = "PORT_UNUSED", LPM_COUNTER_component.lpm_type = "LPM_COUNTER", LPM_COUNTER_component.lpm_width = 32; endmodule // ============================================================ // CNX file retrieval info // ============================================================ // Retrieval info: PRIVATE: ACLR NUMERIC "0" // Retrieval info: PRIVATE: ALOAD NUMERIC "0" // Retrieval info: PRIVATE: ASET NUMERIC "0" // Retrieval info: PRIVATE: ASET_ALL1 NUMERIC "1" // Retrieval info: PRIVATE: CLK_EN NUMERIC "0" // Retrieval info: PRIVATE: CNT_EN NUMERIC "0" // Retrieval info: PRIVATE: CarryIn NUMERIC "0" // Retrieval info: PRIVATE: CarryOut NUMERIC "0" // Retrieval info: PRIVATE: Direction NUMERIC "0" // Retrieval info: PRIVATE: INTENDED_DEVICE_FAMILY STRING "Cyclone V" // Retrieval info: PRIVATE: ModulusCounter NUMERIC "0" // Retrieval info: PRIVATE: ModulusValue NUMERIC "0" // Retrieval info: PRIVATE: SCLR NUMERIC "1" // Retrieval info: PRIVATE: SLOAD NUMERIC "0" // Retrieval info: PRIVATE: SSET NUMERIC "0" // Retrieval info: PRIVATE: SSET_ALL1 NUMERIC "1" // Retrieval info: PRIVATE: SYNTH_WRAPPER_GEN_POSTFIX STRING "0" // Retrieval info: PRIVATE: nBit NUMERIC "32" // Retrieval info: PRIVATE: new_diagram STRING "1" // Retrieval info: LIBRARY: lpm lpm.lpm_components.all // Retrieval info: CONSTANT: LPM_DIRECTION STRING "UP" // Retrieval info: CONSTANT: LPM_PORT_UPDOWN STRING "PORT_UNUSED" // Retrieval info: CONSTANT: LPM_TYPE STRING "LPM_COUNTER" // Retrieval info: CONSTANT: LPM_WIDTH NUMERIC "32" // Retrieval info: USED_PORT: clock 0 0 0 0 INPUT NODEFVAL "clock" // Retrieval info: USED_PORT: q 0 0 32 0 OUTPUT NODEFVAL "q[31..0]" // Retrieval info: USED_PORT: sclr 0 0 0 0 INPUT NODEFVAL "sclr" // Retrieval info: CONNECT: @clock 0 0 0 0 clock 0 0 0 0 // Retrieval info: CONNECT: @sclr 0 0 0 0 sclr 0 0 0 0 // Retrieval info: CONNECT: q 0 0 32 0 @q 0 0 32 0 // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter1.v TRUE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter1.inc FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter1.cmp FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter1.bsf FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter1_inst.v FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter1_bb.v TRUE // Retrieval info: LIB_FILE: lpm

part7_counter2.v

// megafunction wizard: %LPM_COUNTER% // GENERATION: STANDARD // VERSION: WM1.0 // MODULE: LPM_COUNTER // ============================================================ // File Name: part7_counter2.v // Megafunction Name(s): // LPM_COUNTER // // Simulation Library Files(s): // lpm // ============================================================ // ************************************************************ // THIS IS A WIZARD-GENERATED FILE. DO NOT EDIT THIS FILE! // // 13.1.0 Build 162 10/23/2013 SJ Full Version // ************************************************************ //Copyright (C) 1991-2013 Altera Corporation //Your use of Altera Corporation's design tools, logic functions //and other software and tools, and its AMPP partner logic //functions, and any output files from any of the foregoing //(including device programming or simulation files), and any //associated documentation or information are expressly subject //to the terms and conditions of the Altera Program License //Subscription Agreement, Altera MegaCore Function License //Agreement, or other applicable license agreement, including, //without limitation, that your use is for the sole purpose of //programming logic devices manufactured by Altera and sold by //Altera or its authorized distributors. Please refer to the //applicable agreement for further details. // synopsys translate_off `timescale 1 ps / 1 ps // synopsys translate_on module part7_counter2 ( clock, sclr, q); input clock; input sclr; output [31:0] q; wire [31:0] sub_wire0; wire [31:0] q = sub_wire0[31:0]; lpm_counter LPM_COUNTER_component ( .clock (clock), .sclr (sclr), .q (sub_wire0), .aclr (1'b0), .aload (1'b0), .aset (1'b0), .cin (1'b1), .clk_en (1'b1), .cnt_en (1'b1), .cout (), .data ({32{1'b0}}), .eq (), .sload (1'b0), .sset (1'b0), .updown (1'b1)); defparam LPM_COUNTER_component.lpm_direction = "UP", LPM_COUNTER_component.lpm_port_updown = "PORT_UNUSED", LPM_COUNTER_component.lpm_type = "LPM_COUNTER", LPM_COUNTER_component.lpm_width = 32; endmodule // ============================================================ // CNX file retrieval info // ============================================================ // Retrieval info: PRIVATE: ACLR NUMERIC "0" // Retrieval info: PRIVATE: ALOAD NUMERIC "0" // Retrieval info: PRIVATE: ASET NUMERIC "0" // Retrieval info: PRIVATE: ASET_ALL1 NUMERIC "1" // Retrieval info: PRIVATE: CLK_EN NUMERIC "0" // Retrieval info: PRIVATE: CNT_EN NUMERIC "0" // Retrieval info: PRIVATE: CarryIn NUMERIC "0" // Retrieval info: PRIVATE: CarryOut NUMERIC "0" // Retrieval info: PRIVATE: Direction NUMERIC "0" // Retrieval info: PRIVATE: INTENDED_DEVICE_FAMILY STRING "Cyclone V" // Retrieval info: PRIVATE: ModulusCounter NUMERIC "0" // Retrieval info: PRIVATE: ModulusValue NUMERIC "0" // Retrieval info: PRIVATE: SCLR NUMERIC "1" // Retrieval info: PRIVATE: SLOAD NUMERIC "0" // Retrieval info: PRIVATE: SSET NUMERIC "0" // Retrieval info: PRIVATE: SSET_ALL1 NUMERIC "1" // Retrieval info: PRIVATE: SYNTH_WRAPPER_GEN_POSTFIX STRING "0" // Retrieval info: PRIVATE: nBit NUMERIC "32" // Retrieval info: PRIVATE: new_diagram STRING "1" // Retrieval info: LIBRARY: lpm lpm.lpm_components.all // Retrieval info: CONSTANT: LPM_DIRECTION STRING "UP" // Retrieval info: CONSTANT: LPM_PORT_UPDOWN STRING "PORT_UNUSED" // Retrieval info: CONSTANT: LPM_TYPE STRING "LPM_COUNTER" // Retrieval info: CONSTANT: LPM_WIDTH NUMERIC "32" // Retrieval info: USED_PORT: clock 0 0 0 0 INPUT NODEFVAL "clock" // Retrieval info: USED_PORT: q 0 0 32 0 OUTPUT NODEFVAL "q[31..0]" // Retrieval info: USED_PORT: sclr 0 0 0 0 INPUT NODEFVAL "sclr" // Retrieval info: CONNECT: @clock 0 0 0 0 clock 0 0 0 0 // Retrieval info: CONNECT: @sclr 0 0 0 0 sclr 0 0 0 0 // Retrieval info: CONNECT: q 0 0 32 0 @q 0 0 32 0 // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter2.v TRUE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter2.inc FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter2.cmp FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter2.bsf FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter2_inst.v FALSE // Retrieval info: GEN_FILE: TYPE_NORMAL part7_counter2_bb.v TRUE // Retrieval info: LIB_FILE: lpm